# Module Assembly Sequence

# (03/15/01)

# On Alignment Fixture

________ 1.1. Check Shell Length

________ 1.2. Check Assembly frame: Length & straightness

________ 1.3. Tape shell edge for HV insulation

________ 1.4. Install Shell in Assembly frame & retain with Shell sleeve

________ 1.5. Install Dividers and Survey Straws in Shell

________ 1.6. Survey Shell straightness with Divider & Survey Straw

________ 1.7. Remove divider & clean up with vacuum  & alcohol

#### On Optical Table

________ 2.1. Insert fiberglass Cooling Rods

________ 2.2. Install Radiator, Divider & Retaining foam. Pin each divider after insertion

________ 2.3. Install Radiator Retaining Rods

- Rod should not interfere with straw holes

- Retaining foam should be as flat as possible

### ________ 3.1. Remove Shell Sleeve

________ 3.2. Install HV plate

- HV plate should have wicking sheet.

________ 3.3. Install HV plate washer stopper.

________ 3.4. HV test: 3KV whole module, both sides (test HV plate vs shell)

________ 3.5. Insert steel alignment rods (~10 evenly distributed in module)

________ 3.6. Insert Straws

- Alternate insertion direction

- Green MARK goes to BACK side of module

- use bullets with short straw extensions except edges and near corners

- Never pull a straw backward

- Never put a straw into a hole if a rod (plastic or steel) is not fully pulled out

________ 3.7. Inspect straw ends for broken carbon fiber and remove radiator fiber

##### On Alignment Fixture

________ 4.1. Survey Survey Straws

________ 4.2. Replace Survey straws with real straws

________ 4.3. HV test: 2.5KV island scan

________ 4.4. Super glue HV plate to Shell

- HV plate should be flush to stopper

- Push shell against HV plate when gluing, so that it doesn’t pop out

#### On Optical Table

________ 5.1. Attach straw stopper plate at front side

- Make sure straw projections at both ends are even.

________ 5.2. Conductive glue Endplug of BACK side first (tra-duct 2902)

- Do not let conductive glue get to the inner edge of endplug

- Face the overlap section of conductive glue to HV island

- Minimize glue outside of HV island

- Let cure overnight

________ 5.3. Remove straw stopper plate from front side.

- Check that conductive glue has cured before removing plate.

- If have to re-apply straw stopper plate, do not tighten with screw.

________ 5.4. Straw resistance measurement

________ 5.5. HV test: 2.5KV island scan

###### On Alignment Fixture

________ 6.1. Attach straw stopper plate at back side use triangle shims to raise ~2mm gap.

________ 6.2. Conductive glue Endplug of Front side		(tra-duct 2902)

- Let cure overnight

#### On Optical Table

________ 7.1. Clean excess glue, (Insert pin in socket, try uv glue)

________ 7.2. HV Island resistance measurement

________ 7.3. HV test: 2.5KV island scan

________ 7.4. Seal straw  at both ends				(Araldite 2019)(AY103)

- Mix glue 45 min before use

- Glue tip should go around the straw end

- glue around edge of  thin plate

- coat HV plate wall up to tension plate ledge but not exceeding the ledge

________ 7.5. Clean gas fitting seating before glue harden

________ 7.6. Glue Divider Ears 					(Araldite 2011)

- Do not force needle in if encounter resistance – may poke hole on straw

________ 7.7. Remove pin when AY103 is half cure

- Vacuum sockets to remove glue

###### On Alignment Fixture

- Let cure overnight

- Check every 15 min for ~1 hours to make sure no excessive glue at bottom

________ 7.8. Cut divider ears and remove module from assembly frame

#### On Optical Table

________ 8.1. Super glue shell to HV plate & seal HV plate to Shell with UV glue.

- Insert Silicone tube in cooling tube holes and seal with UV glue

#### On Potting Jig

________ 9.1. Pot back side						(Stycast 1266)

- heat HV plate ( monitor temperature with thermister ~50C )

- Don’t let retaining rods touch the glue

- Watch out for glue overflow @ straws & @ kapton flex

- Type 2 mix 37.46+10.49 = 47.95 gm

- Type 3 mix 58.21+16.30 = 74.51 gm

- Let cure overnight

________ 9.2. Pot front side						(Stycast 1266)

- Let cure overnight

#### On Optical Table

________ 10.1. Check cooling tube hole with rods, and clean with 1/8” rimmer

________ 10.2. Check capacitor sockets with pin, and clean with #76 drill

________ 10.3. Clean gas fitting seat with #31 drill, check that gas fitting fits under tension plate.

________ 10.4. Final HV test: 2.5KV island scan

________ 10.5. Leak test with dry air inside and outside (Use UV glue to seal plugs)

________ 10.6. Clean UV glue and rinse/wipe with alcohol

________ 10.7. Attach Tension plate & components

________ 10.8. Check capacitor pin continuity

- From kapton to capacitor pin (4.7K ohm)

________ 10.9. Super glue Tension plate at top and bottom edges (side A & C, 3 drops each side)

- Make sure tension plate goes all the way in to the ledge.

#### Move to stringing station

________ 11.1. Prepare sense wire for stringing.

________ 11.2. String sense wire into module.

- 11.1. Use 70gm weight. Tension cut off: 135Hz (55.5g)  – 165Hz (83g)

- 11.2. HV test 1800V after stringing (require I < 10uA)

- 11.3. Rework channels failed HV test

#### Remove from stringing station

________ 12.1. Wire tension measurement @16-channels station

________ 12.2. Seal tension plate edges and eyelet / taper pin with AY103

________ 12.3. Leak test

________ 12.4. Wire signal test

________ 12.5. Wire re-work + tension/leak/signal test

________ 12.6. Wire gain measurement	(Hampton)